EXP NO. 16

SIMULATION AND ANALYSIS OF UNIFORM AND NON-UNIFORM QUANTIZATION USING MATLAB

PROGRAM

clc;

clear;

close all;

% Input Signal

t = 0:0.001:1;

x = sin(2*pi*5*t);

% Uniform Quantization (8 Levels)

L = 8;

x_uni = round((x+1)*(L-1)/2);

x_uni = (x_uni*2/(L-1))-1;

% Non-Uniform Quantization (mu-law)

mu = 255;

% Compression

x_comp = sign(x).*log(1+mu*abs(x))/log(1+mu);

% Quantization

x_non = round((x_comp+1)*(L-1)/2);

x_non = (x_non*2/(L-1))-1;

% Expansion

x_non = sign(x_non).* ...

((1+mu).^abs(x_non)-1)/mu;

% Plot Results

figure;

subplot(3,1,1)

plot(t,x,'b')

title('Original Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(3,1,2)

stairs(t,x_uni,'r')

title('Uniform Quantization')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(3,1,3)

stairs(t,x_non,'g')

title('Non-Uniform Quantization (\mu-law)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

OUTPUT